# Smart AutoRoof Weather Monitoring System.

Domain: Embedded Systems .

Submitted By:
Harsha Vardini.
N.Sri Sai Mani Venkata Reddy.
Naseeb.
S20240020285
S20240020314
S20240020315
Department: ECE 
 IIIT Sricity.

Guided By:
 Dr.Paul Brainerd.

# 2.Outline

- Introduction

- Literature Review

- Objectives

- Components & Specifications

- Work done

- Methodology

- Arduino Code Overview

- Results and Discussions

- Conclusion & Future Scope

- References.

## 3.Introduction:

### Problem Statement

Traditional weather measurements rely on manual observation and standalone instruments. They lack:

- Real-time monitoring

- Remote access

- Automated alerts

- Data logging

### Proposed Solution

This project builds a Smart IoT-based Weather Monitoring System that senses and uploads weather parameters to the Blynk Cloud in real-time. The system uses:

- DHT11 for temperature & humidity

- BMP180 for atmospheric pressure

- Rain Sensor for rain intensity

- LDR for light intensity

- Servo Motor & LEDs for automated actions

- NodeMCU ESP8266 for Wi-Fi & Cloud connectivity

### Key Features

- Weather data viewed live from anywhere using Blynk App

- Automated gate/cover movement using servo during rain

- LED indicators for safety and weather conditions

- Real-time notifications

## 4. Literature Review:

### Existing Weather Monitoring Techniques

| Method | Limitations |
| --- | --- |
| Manual observation | Time-consuming, error-prone |
| Wired digital sensors | No remote monitoring |
| Bluetooth weather stations | Limited range (10–15 m) |
| Commercial IoT stations | Very expensive |

### Research Findings

- Studies show IoT improves data accuracy and accessibility.

- DHT11 and BMP180 sensors are widely used for low-cost weather systems.

- Servo-based automation is increasingly used for protective covers, greenhouse panels, etc.

### Gap Identified

Many existing IoT systems only measure data but do not perform automatic actions.
 Our project fills this gap by adding servo + LED automation.

## 5. Objectives:

- To design a fully IoT-enabled weather monitoring station

- Provide real-time data logging on a cloud platform

- Enable smartphone access from anywhere

- Implement automatic actuation based on rain conditions

- Create a low-cost, power-efficient design

- Achieve reliability under continuous 24/7 operation

- Use minimal hardware components.

## 6. Components Used & Specifications

### 1. NodeMCU ESP8266

- Wi-Fi enabled microcontroller

- Operating voltage: 3.3V

- Role: Connects system to Blynk Cloud

### 2. DHT11 Sensor

- Measures temperature & humidity

- Temp range: 0–50°C

- Humidity range: 20–90%

### 3. BMP180 Pressure Sensor

- High precision barometric pressure sensor

- Uses I2C communication

- Role: Pressure & altitude

### 4. Rain Sensor

- Outputs analog value 0–1023mm.

- Used for rain detection

### 5. LDR (Light Dependent Resistor)

- Detects light/dark

- Digital output used

### 6. Servo Motor (SG90)

- Rotates 0–180°

- Used for gate/shed automation

### 7. LEDs

- LED1 (Rain detected)

- LED2 (No rain)

## 7. Methodology

### A. System Architecture

The system senses weather parameters and uploads them to cloud via NodeMCU. Physical actions (servo & LEDs) respond automatically based on conditions.

### B. Circuit Design

- DHT11 connected to D5

- BMP180 connected via I2C (D1 = SCL, D2 = SDA)

- Rain Sensor → A0

- LDR → D0

- LEDs → D4 & D7

- Servo → D6

- NodeMCU → Wi-Fi + Cloud

### C. Working Process

- Sensors continuously read weather conditions

- NodeMCU uploads data to Blynk cloud

- Blynk app displays live data:

  - Temperature

  - Humidity

  - Pressure

  - Rain intensity

  - Light intensity

- Automation Logic:

  - If rain detected → Servo closes gate, LED1 ON

  - If no rain → Servo opens gate, LED2 ON

### D. Blynk IoT Configuration

- Created template

- Added Virtual Pins V0–V4

- Real-time data charts displayed

- Device connected via Auth Token

### E. User Dashboard

Users can:

- Monitor values real-time

- Visualize graphs

- Get alerts

# 8. Algorithm / Flow Chart

#### Algorithm(Code for NodeMCU)(written in Arduino IDE):

// ---- Blynk Template Info ----

#define BLYNK_TEMPLATE_ID        "TMPL3czG1h-TZ"

#define BLYNK_TEMPLATE_NAME      "smart weather forecast"

#define BLYNK_AUTH_TOKEN         "CkRSKcXYFtqmqaDxz3jUgw8hH-KvchnO"

#include <Wire.h>

#include <Adafruit_BMP085.h>

#include <DHT.h>

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

#include <Servo.h>

// ---- PIN DEFINITIONS ----

#define DHTPIN D5

#define DHTTYPE DHT11

#define RAIN_ANALOG A0

#define LDR_PIN D0       // OK

#define SERVO_PIN D6     // Servo on D6

#define LED_PRESS D4     // Boot-safe HIGH

#define LED_TEMP D7

// ---- WIFI ----

char ssid[] = "Infinix..94677";

char pass[] = "sindhu000";

// ---- SENSOR OBJECTS ----

Adafruit_BMP085 bmp;

DHT dht(DHTPIN, DHTTYPE);

Servo myservo;

float temperature, humidity, pressure;

int rainValue;

int ldrValue;

bool servoAt180 = false;   // servo state

bool servoAt0 = true;      // default starting state

void setup() {

Serial.begin(115200);

delay(500);

// WiFi

Serial.println("Connecting to WiFi...");

WiFi.begin(ssid, pass);

while (WiFi.status() != WL_CONNECTED) {

Serial.print(".");

delay(500);

}

Serial.println("\nWiFi Connected!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

// Blynk

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

// Sensors

Wire.begin(D2, D1);  // SDA=D2 , SCL=D1

if (!bmp.begin()) Serial.println("BMP085 sensor not found!");

dht.begin();

// Servo

myservo.attach(SERVO_PIN);   // D6

// LEDs

pinMode(LED_PRESS, OUTPUT);

pinMode(LED_TEMP, OUTPUT);

Serial.println("Weather Station Started...");

}

void loop() {

Blynk.run();

// --- Read Sensors ---

temperature = dht.readTemperature();

humidity = dht.readHumidity();

pressure = bmp.readPressure() / 100.0;

rainValue = analogRead(RAIN_ANALOG);

ldrValue = analogRead(LDR_PIN);

// --- LDR Threshold ---

int ldrStatus = (ldrValue > 500) ? 1 : 0;

// --- Servo Based on Rain Value ---

int rainThreshold = 100;

if (rainValue > rainThreshold && !servoAt180) {

// Rotate FORWARD to 180 degree

Serial.println("Rain > 100 → Servo turning to 180°");

myservo.write(180);

delay(700);   // allow motion

myservo.write(90);  // STOP torque

servoAt180 = true;

servoAt0 = false;

}

if (rainValue <= rainThreshold && !servoAt0) {

// Rotate BACKWARD to 0 degree

Serial.println("Rain <= 100 → Servo returning to 0°");

myservo.write(0);

delay(700);   // allow motion

myservo.write(90);  // STOP torque

servoAt0 = true;

servoAt180 = false;

}

// --- Pressure LED ---

digitalWrite(LED_PRESS, pressure > 1000);

// --- Temperature LED ---

digitalWrite(LED_TEMP, temperature > 40);

// --- Send Data to Blynk ---

Blynk.virtualWrite(V0, temperature);

Blynk.virtualWrite(V1, humidity);

Blynk.virtualWrite(V2, pressure);

Blynk.virtualWrite(V3, rainValue);

Blynk.virtualWrite(V4, ldrStatus);

// --- Serial Monitor ---

Serial.print("Temp: "); Serial.print(temperature);

Serial.print("  Hum: "); Serial.print(humidity);

Serial.print("  Pressure: "); Serial.print(pressure);

Serial.print("  Rain: "); Serial.print(rainValue);

Serial.print("  Servo: "); Serial.print(servoAt180 ? "180°" : "0°");

Serial.print("  LDR: "); Serial.println(ldrStatus);

delay(400);

}

#### Flow Chart:

Start
 |
Initialize Wi-Fi, Blynk, Sensors
 |
Read temperature, humidity, pressure
 |
Read rain & LDR values
 |
Send all values to Blynk
 |
Check rain condition:
    If rain < threshold:
          Close gate
          LED1 ON
    Else:
          Open gate
          LED2 ON
 |
Wait 1 second
 |
Loop back

# 9. Results & Discussion:

### 1. Sensor Performance

| Parameter | Observed Result |
| --- | --- |
| Temperature | Accurate ±1°C |
| Humidity | Stable readings |
| Pressure | Highly accurate (±1 hPa) |
| Rain Sensor | Responds instantly |
| LDR | Fast switching |

### 2. Automation Response

- Servo motor responded within 0.5 seconds

- Rain-based automation worked smoothly

- LED indicators clearly showed system status

### 3. IoT Performance

- Blynk app updated data every second

- No delay in cloud transmission

- System worked for 12 hours continuously without reset

### 4. Observational Readings Sample

| Temp | Hum | Pressure | Light |
| --- | --- | --- | --- |
| 28°C | 60% | 1004 hPa | Day |
| 27°C | 62% | 1002 hPa | Night (LED ON) |
| 29°C | 57% | 1005 hPa | Day |

# 10. Conclusion

The Smart Weather Monitoring System was successfully implemented with IoT connectivity and automation features. The system accurately measures key weather parameters and can be monitored remotely. Automatic gate movement and LED indicators enhance practical usability.

The project is cost-effective and scalable, making it suitable for agricultural, residential, and industrial applications.

# 11. References

- ESP8266 Technical Documentation NodeMCU ESP8266 Pinout, Specifications, Features & Datasheet

- Arduino IDE Reference Arduino IDE | Arduino Documentation

- Blynk IoT Cloud Documentation(Introduction | Blynk Documentation)

- DHT11, BMP180 Datasheets 
         DHT11 Humidity & Temperature Sensor
        BMP180 Datasheet by SparkFun Electronics | Digi-Key Electronics

<< THANK YOU>>